## ELEC 4320 Final Project Proposal: Enhanced Real-Time Stereo Depth Estimation with Pseudo-Color and Ranging

### 1. Introduction and Project Goal

This project proposes the design and implementation of an advanced real-time stereo vision system on the Basys3 FPGA platform, utilizing dual OV7670 cameras for depth estimation. The core goal is to significantly enhance the algorithmic robustness, visualization quality, and application features of a open-source depth map project (https://github.com/Archfx/FPGA_depthMap), ensuring the design meets the originality and complexity requirements for the course.

The system will transition from a basic black-and-white depth map display to a high-utility ranging system with modern visualization.

### 2. Proposed Design Enhancements (Innovation & Complexity)

The existing baseline project typically uses the resource-efficient but low-robustness Sum of Absolute Differences (SAD) algorithm. To increase technical difficulty and algorithmic quality, the following core modules will be designed and integrated in Verilog:

| Feature | Baseline Project (SAD/Grayscale) | Proposed Enhancement | Technical Difficulty & Project Merit |
| --- | --- | --- | --- |
| Stereo Matching Core | Sum of Absolute Differences (SAD) | Census Transform & Hamming Distance | High. Requires designing a register-based windowing module (3x3 or 5x5) and a fully pipelined Hamming Weight/Popcount circuit. This provides robust matching against brightness variations, a key weakness of the baseline. |
| Image Output | Grayscale Depth Map (0-255) | Pseudo-Color Visualization (Heatmap) | Medium. Requires a small Read-Only Memory (ROM) or Look-Up Table (LUT) to map the 8-bit disparity value to 12-bit RGB color codes (e.g., Red=Near, Blue=Far). This vastly improves visual interpretation. |
| Application Layer | Passive Display | Real-Time Digital Ranging | Medium. Implements a disparity-to-distance conversion (Z=f B/ d) and a Binary-to-BCD module to display the distance (in cm) of the object at the center of the frame onto the Basys3 4-digit 7-segment display. |
| Image Quality | Raw Disparity Output | 3x3 Median Filtering (Post processing) | High. Requires a 3x3 sliding window (using BRAM as line buffers) and a parallel sorting network to calculate the median, effectively eliminating "salt and pepper" noise from the depth map. |

### 3. Core Module Elaboration and I/O Utilization

The design will be divided into the following key modules, fully satisfying the functional requirements:

| Module Name | Functionality | On-Chip Memory / Sequential Logic | I/O Utilization |
| --- | --- | --- | --- |
| Camera / VGA Drivers | Interfaces dual OV7670 to receive data and outputs to a VGA monitor. | Line Buffers (BRAM access) | Output: VGA Signal (Hsync, Vsync, RGB) |
| Census Matching Core | Generates bit strings and computes Hamming Distance for disparity. | Registers for 3x3/5x5 windowing. | N/A |
| Distance Ranging FSM | Implements the Finite State Machine (FSM) to control the depth processing pipeline, calculate distance, and manage the multi-stage 7-segment display refresh. | Control Registers | Input: Center Button (BTN_C) to freeze / unfreeze the distance. |
| Output Module | Maps disparity to pseudo-color and drives the 7-segment display. | ROM/LUT for Color Mapping. | Output: 7-Segment Display, LEDs for status. |
| User Input | Selects operating mode (Raw Disparity, Filtered Disparity, or Ranging). | Registers for mode selection. | Input: DIP Switches (SW[1:0]) for mode select. |

### 4. Technical Difficulty Evaluation

- Algorithmic Complexity: The shift from SAD (simple subtraction/addition) to Census Transform + Hamming Distance significantly increases the logic complexity, requiring a deeper understanding of image processing and highly optimized bit-manipulation logic in Verilog.

- Pipelining and Timing: The Basys3's Artix-7 FPGA is resource-constrained. Achieving real-time frame rates (e.g., 30 FPS) with the complex Census and optional Median Filter modules will necessitate heavy pipelining and optimization to prevent timing violations, which is a major design challenge.

- Resource Management: Efficient management of the limited on-chip BRAM is critical for the necessary line buffers and ROMs, requiring careful memory partitioning and addressing logic.

- I/O Integration: The project utilizes four distinct I/O components (Dual OV7670, VGA, Switches/Buttons, 7-Segment Display), demonstrating high interaction and full utilization of the Basys3 peripherals.

### 5. Project Timeline

| Task | Oct. 31 - Nov. 15 | Nov. 16 - Nov. 30 | Dec. 1 - Dec. 15 | Dec. 16/17 |
| --- | --- | --- | --- | --- |
| Core Modules | Camera/VGA Drivers Adaptation & Test. | Census Transform Core Design & Simulation. | Median Filter / Post-processing integration. | Final Debugging & Demo Prep. |
| Feature Modules | Pseudo-Color LUT Design. Binary-to-BCD/7-Seg Driver Design. | FSM Integration & Control Logic (Mode switching). | Ranging Formula Implementation & Calibration. | Presentation & Demo. |
| Documentation | Proposal Submission. | Module Documentation (Schematics, I/O). | Final Report Compilation and Citation. | Final Submission. |